Quick Start for Firmware developers

This section serves the purpose to help future developers to setup their development environment on their PC, get familiar with the application code structure. And finally being able to modify the code and debug on the CC2640 Bluetooth module developed for the company.

For convenience reason, CCS (TI’s eclipse based IDE) is used only for editing source code.

And for the high compiling speed and quality, the IAR 7.50 is used for compiling and generating binary files.

Setup the IDE

For initial setup follow through 2.5 Setting up the Integrated Development Environment in swru393b.pdf

Two things to keep in mind

- Install IAR 7.50 or above to a path with no space, no Chinese characters

- Download the BLE-STACK-2-1-1 and install it to the default location which is C:\ti\simplelink\ble_cc26xx_2_01_01_44627

- Verify the Custom Argument Variables for the Workspace are the same as shown below

Install the Flash Programmer 2

This windows application can be used to flash firmware to the CC2640 for the first time when connected to a debugger like XDS100V3

http://www.ti.com/tool/flash-programmer

Setup the Sensor controller studio

The sensor controller module is used for AD sampling in the background in the application independent of the main processor to save processing time of the main CPU.

Get Sensor controller studio from TI and install it

http://www.ti.com/tool/sensor-controller-studio

To create a Sensor Controller Studio (SCS) project from scratch

- After that create a Sensor Controller project named RGZ Worker, and a task named AD Scan

- Create 3 ADC pins

- Enable RTC-Based Execution Scheduling

Add one constant

Add two structures

Edit the source code starting from the initialization code

// Schedule the first execution fwScheduleTask(1);

Execution Code

// Enable the ADC (3.3V reference, 2.7 us sample time, manual trigger) adcEnableSync(ADC_REF_VDDS_REL, ADC_SAMPLE_TIME_2P7_US, ADC_TRIGGER_MANUAL);//ADC_REF_FIXED // For each pin (with one entry per pin in cfg.pAdcValue[] and output.pAdcValue[]) ... for (U16 n = 0; n < ADC_COUNT; n++) { // Select ADC input adcSelectGpioInput(cfg.pAuxAD[n]); // Sample the pin and store the ADC value adcGenManualTrigger(); adcReadFifo(output.pSamples[n]); } // Disable the ADC adcDisable(); // Schedule the next execution fwScheduleTask(1);

Leave the Termination Code empty

I/O Mapping, this need to match with the actual hardware configuration

Finally generate the source code for the application

Alternatively we can just open up the Sensor Controller project file rgz_worker.scp provided

Modified source code

After setting up the IDE, some modification need to be made to the source code

Board file

This will define pins and peripherals for our customer board

Read section

Off chip OAD flash related files

This will define the SPI flash IC page layout, device ID, manufacturer ID etc.

Read section

OAD source code changes

Improved fail safe for off chip OAD

Read section

Add TI RTOS GPTimer Driver

The GPTimer Driver is not included in the driver folder by default, we will use the pre-released driver

Read section

Notes on the software

Difference compared with legacy code

How the BLE AT commands are received

simpleBLEPeripheral.c

Line 1353

ArrInfo response_ArrInfo = ATControl(pWrite->data, getUpdateCmdVariableLength());

The ATControl take the input string and compare with known AT command stored inside an array of structure, and execute functions if any match were found.

How the BLE commands are send as AT command

An immediate response message can be send by returning an “ArrInfo” struct from the ATControl function call.

Or we can send arbitrary data by calling SendDataBLE function which take a “ArrInfo” struct as parameter. The message will be send asynchronously by the Simple BLE Peripheral (SBP) task.

About pin assignment

All pin assignment are defined in the custom Board.h file

About loops

The legacy loops relaying on the system tick are moved to the PeriodicTimer.c since the system tick of CC2640 is running at 1ms while the legacy system tick period is much lower.

About receiving One Wire message

Since receiving one wire message require much higher IO scanning rate, a hardware Timer0A is used for this purpose. In the Timer ISR, it queues detected level change and its duration inside a buffer. The PeriodicTimer task will read out the queue asynchronously every 100ms.

About ADC

We use sensor control to handle the ADC scan to offload the processing resource from main CPU to sensor controller.

The AD pins configuration are defined in _Parameter.h

#if BOARD_VERSION == CC2640_RGZ_A #define AUXIO_A_LV 3 #define AUXIO_A_LV_48VSW 7 #define AUXIO_A_OCJ 4 #elif(BOARD_VERSION==CC2640_RGZ_B) #define AUXIO_A_LV 5 #define AUXIO_A_LV_48VSW 7 #define AUXIO_A_OCJ 6 #endif

The number need to match with ones defined in the Sensor Controller Studio once IO is mapped

About storing persistent data to on board Flash

Persistent data are stored in the SNV section of the on board Flash

Since the “osal_snv_write” and “osal_snv_read” functions can only be called within a task which registered ICall like the SBP task. So in the implementation of the FlashSNV.c we use callback to notify the SBP task to do those operations for us by calling “NotifyFlashAction” function.

SW Debug Note

OAD

Off-Chip OAD

Forum Q&A

Adjusting Off-chip Flash Memory Layout

C:\ti\simplelink\ble_cc26xx_2_01_01_44627\Projects\ble\Profiles\OAD\CC26xx\ext_flash_layout.h

Change the value to match with our 2M-bit Flash W25X20CLUXIG (256KB)

4M-bit (512KB) W25X40CL  0x78000

2M-bit (256KB) W25X20CL  0x3F000

1M-bit (128kB) W25X10CL  0x1F000

// Image information (meta-data) #define EFL_ADDR_META 0x3F000 //0x78000

Change Flash Manufacturer and Device Identification

Refer to Flash IC Datasheet

e.g. in section 8.2.1 Manufacturer and Device Identification of the W25X20CLUXIG’s datasheet we have

C:\ti\simplelink\ble_cc26xx_2_01_01_44627\Projects\ble\util\BIM_extflash\CC26xx\Source\CC26XXST_0120\ bsp.h

And

/* External flash manufacturer and device ID */ #define EXT_FLASH_MAN_ID 0xEF// Winbond Serial Flash #define EXT_FLASH_DEV_ID 0x11//0x12 //for w25x20cl it is 0x11, for w25x40cl it is 0x12

Build BIM_extflash (BIM_ext.hex)

Open up the IAR workspace

BIM_extflash.eww

Note: use the FlashOnly configuration by default, not the sensortag configuration

File location

C:\ti\simplelink\ble_cc26xx_2_01_01_44627\Projects\ble\util\BIM_extflash\CC26xx\IAR\BIM_extflash.eww

To change SPI pins, open up bsp.h and change the 4 pins’ assignment accordingly

The file location

C:\ti\simplelink\ble_cc26xx_2_01_01_44627\Projects\ble\util\BIM_extflash\CC26xx\Source\CC26XXST_0120

Those pins are referenced in file bsp_spi.c

Application-Stack combined image generated by IAR

FlashOnly_OAD_ST_ExtFlash\Exe\OAD_FULL_IMAGE.hex

ExtFlash OAD image(OAD_FULL_IMAGE.hex)

Open up SBP project

Edit the SPI pins in Board.h file, add other Marcos as shown bellow

#define Board_SPI0_MISO IOID_28 #define Board_SPI0_MOSI IOID_30 #define Board_SPI0_CLK IOID_29 #define Board_SPI0_CSN IOID_27 /* External flash manufacturer and device ID */ #define EXT_FLASH_MAN_ID 0xEF// Winbond Serial Flash #define EXT_FLASH_DEV_ID 0x11//0x12 //for w25x20cl it is 0x11, for w25x40cl it is 0x12 /* SPI */ #define Board_SPI_FLASH_CS Board_SPI0_CSN #define Board_FLASH_CS_ON 0 #define Board_FLASH_CS_OFF 1 #define Board_DEVPK_CS_ON 1 #define Board_DEVPK_CS_OFF 0

Switch to FlashOnly_OAD_ST_ExtFlash configuration

We have modified the Additional include directories.  We added sensor controller generated source code directory and modified the Board files directory to our Board.c/h files.

$PROJ_DIR$ $PROJ_DIR$/../../../Source/Application $PROJ_DIR$/../../../Source/SC_Src $PROJ_DIR$/../../../../../../../Projects/ble/Include $PROJ_DIR$/../../../../../../../Projects/ble/ICall/Include $PROJ_DIR$/../../../../../../../Projects/ble/Profiles/Roles/CC26xx $PROJ_DIR$/../../../../../../../Projects/ble/Profiles/Roles $PROJ_DIR$/../../../../../../../Projects/ble/Profiles/DevInfo $PROJ_DIR$/../../../../../../../Projects/ble/Profiles/SimpleProfile/CC26xx $PROJ_DIR$/../../../../../../../Projects/ble/Profiles/SimpleProfile $PROJ_DIR$/../../../../../../../Projects/ble/Profiles/OAD/CC26xx $PROJ_DIR$/../../../../../../../Projects/ble/common/cc26xx $PROJ_DIR$/../../../../../../../Components/applib/heap $PROJ_DIR$/../../../../../../../Components/ble/hci $PROJ_DIR$/../../../../../../../Components/ble/controller/CC26xx/include $PROJ_DIR$/../../../../../../../Components/ble/host $PROJ_DIR$/../../../../../../../Components/hal/target/CC2650/rom $PROJ_DIR$/../../../../../../../Components/hal/target/CC2650TIRTOS $PROJ_DIR$/../../../../../../../Components/hal/target/_common/cc26xx $PROJ_DIR$/../../../../../../../Components/hal/include $PROJ_DIR$/../../../../../../../Components/osal/include $PROJ_DIR$/../../../../../../../Components/services/sdata $PROJ_DIR$/../../../../../../../Components/services/saddr $PROJ_DIR$/../../../../../../../Components/icall/include $PROJ_DIR$/../../../../../../../Components/ble/include $CC26XXWARE$ $TI_RTOS_DRIVERS_BASE$ $TI_RTOS_DRIVERS_BASE$\ti\boards\CC2640RGZ\CC2650EM_7ID $PROJ_DIR$/../../../../../SensorTag/CC26xx/Source/Application/Board_patch/interfaces $PROJ_DIR$/../../../../../SensorTag/CC26xx/Source/Application/Board_patch/devices

Make sure we have commented out those two lines in TOOLS/appBLE.cfg

Update the devInfoFirmwareRev inside file PROFILES/devinfoservice.c

static uint8 devInfoFirmwareRev[DEVINFO_STR_ATTR_LEN+1] = "1";

In CCS we have added

"${ORG_PROJ_DIR}/../../../Source/SC_Src"

In the entry

Also replaced

"${TI_RTOS_BOARD_BASE}/CC26XXST_0120"

With

"${TI_RTOS_DRIVERS_BASE}/ti/boards/CC2640RGZ/CC2650EM_7ID"

Generate the Full OAD image OAD_FULL_IMAGE.hex

Have this post build command in IAR

python "C:\Python27\Scripts\hexmerge.py" -o "$PROJ_DIR$\FlashOnly_OAD_ST_ExtFlash\Exe\OAD_FULL_IMAGE.hex" -r "1000:1CFFF" --overlap=error "$PROJ_DIR$\FlashOnly_OAD_ST_ExtFlash\Exe\SimpleBLEPeripheral_OADExtFlash.hex":1000:EFFF "$PROJ_DIR$\..\..\Stack\CC2640\FlashROM\Exe\SimpleBLEPeripheralStackFlashROM.hex":F000:1CFFF

Flash Super image

We can generate a super image which combines the two hex file (Stack,App + BIM)

This Post Build script will generate

OFFCHIP_APP_OAD.bin for Mobile OAD

OAD_FULL_IMAGE.hex for BLE Device monitor to OAD both Stack and Application

SUPER_FULL_IMAGE.hex to be flashed to chip first time using Flash Programmer 2

This SUPER_FULL_IMAGE.hex file generated bellow need to be flashed to chip first using Flash Programmer 2

@ECHO OFF SET PROJ_DIR=%1 SET BINFILE=OFFCHIP_APP_OAD.bin ECHO project directory is "%PROJ_DIR%" ECHO merge app and stack hex files to one file python "C:\Python27\Scripts\hexmerge.py" -o "%PROJ_DIR%\FlashOnly_OAD_ST_ExtFlash\Exe\OAD_FULL_IMAGE.hex" -r "1000:1CFFF" --overlap=error "%PROJ_DIR%\FlashOnly_OAD_ST_ExtFlash\Exe\SimpleBLEPeripheral_OADExtFlash.hex":1000:EFFF "%PROJ_DIR%\..\..\Stack\CC2640\FlashROM\Exe\SimpleBLEPeripheralStackFlashROM.hex":F000:1CFFF ECHO Generate OAD binary file "%BINFILE%" python "C:\Python27\Scripts\hex2bin.py" -r "1000:BFFF" "%PROJ_DIR%\FlashOnly_OAD_ST_ExtFlash\Exe\SimpleBLEPeripheral_OADExtFlash.hex" "%PROJ_DIR%\FlashOnly_OAD_ST_ExtFlash\Exe\%BINFILE%" ECHO Generate OAD super hex file "SUPER_FULL_IMAGE.hex" python "C:\Python27\Scripts\hexmerge.py" -o "%PROJ_DIR%\FlashOnly_OAD_ST_ExtFlash\Exe\SUPER_FULL_IMAGE.hex" -r "0000:1FFFF" --overlap=error "%PROJ_DIR%\FlashOnly_OAD_ST_ExtFlash\Exe\SimpleBLEPeripheral_OADExtFlash.hex":1000:1EFFF "%PROJ_DIR%\..\..\Stack\CC2640\FlashROM\Exe\SimpleBLEPeripheralStackFlashROM.hex":1000:1EFFF "%PROJ_DIR%\..\..\..\..\..\..\ble\util\BIM_extflash\CC26xx\IAR\FlashOnly\Exe\BIM_ext.hex":0000:1FFFF

Those generated hex files reside in directory

C:\ti\simplelink\ble_cc26xx_2_01_01_44627\Projects\ble\SimpleBLEPeripheral\CC26xx\IAR\Application\CC2640\FlashOnly_OAD_ST_ExtFlash\Exe

Do the OAD

We have change out application code

Updated the devInfoFirmwareRev inside file PROFILES/devinfoservice.c

static uint8 devInfoFirmwareRev[DEVINFO_STR_ATTR_LEN+1] = "2";

And compiled successful, which generated new OAD_FULL_IMAGE.hex file

To do the off chip OAD

- Open up BLE Device Monitor, click here for info to make a HostTest USB dongle

- C:\ti\simplelink\ble_cc26xx_2_01_01_44627\Accessories\HexFiles\ CC2640_SmartRF-7ID_HostTestRelease_All.hex  or C:\ti\simplelink\ble_sdk_2_02_00_31\examples\hex\cc2650lp_host_test_rel.hex

- Update the GAP Settings

- Select the new OAD_FULL_IMAGE.hex or just the application hex SimpleBLEPeripheral_OADExtFlash.hex if the stack is not modified.

- Click start download the code

After the OAD success info, board will reset itself. If the board is not resetting itself, it might be a sign of problem.

Do another scan and connect to our board, verify the board FW was updated, as the Firmware revision shown 2 below

Debug the off chip OAD

We add the cs pin to open

Open will fail here since cs pin is already opened

Solution

In Board.h file

#define Board_SPI0_CSN PIN_UNASSIGNED

Actually there are two slightly different off-chip OAD models. One is used in SensorTag OAD sample application and the other is used in SimpleBLEPeripheral OAD sample application runnng on SensorTag hardware. The OAD User's Guide describes only the latter one which is more fail-safe than the former one. The differences between SensorTag OAD sample app and SBP OAD sample app are as follows: <SensorTag OAD sample app> - Works with the BIM of the configuration 'BIM_extflash - FlashOnly_SensorTag' - The application image starts at 0x00000 - When reset, the application starts. - When OAD completes, the application calls the BIM so that it copies the downloaded image from the off-chip flash to the on-chip flash and reset the system. - Works with Android/iOS SensorTag app for OAD <SBP OAD sample app running on SensorTag HW> - Works with the BIM of the configuration 'BIM_extflash - FlashOnly' - The application image starts at 0x01000 - When reset, the BIM starts and copies the image from the off-chip flash to the on-chip flash if any. Then, jumps to the application image. - When OAD completes, the application reset the system. - Works with Windows BLE Device Monitor application for OAD Hence, which model is more proper for your product should be considered first. Once the model is chosen, please start with the sample application corresponding to the chosen OAD model.

https://e2e.ti.com/support/wireless_connectivity/bluetooth_low_energy/f/538/t/448400

Try use the BIM_extflash – FlashOnly_SensorTag configuration

Try OAD the super image

Try disconnect all debug pins while doing OAD

(1) Config BIM project to FlashOnly_SensorTag (2) DON'T follow the official guide to change .icf file ! You MUST use the original one ! (3) DON'T change APP_IMAGE_START ! (4) Modify something, change FW_VERSION, and re-build all three projects, BIM, App, and Stack. (5) Combine ALL THREE .hex files into one OAD .bin image by using JFlash. (6) Use Android app to do OAD with this .bin image Indeed, I did nothing except for (1) and (5). Yes, that's it.

Finally check if the Sensor controller is using any pins conflict with the Flash SPI pins, otherwise the OAD will fail.

Screenshot of RGZ_B board sensor controller ADC pins assignment

About interrupting OAD process

Interrupting the OAD process by stopping the Android App,

The target need a manual reset before restarting the process again.

This manual reset is needed since the external flash is opened and cannot be reopened again in the official OAD logic. This is a limitation of the official OAD source code, and could be customized to remove the limitation.

The flash open is inside oad_target_externalFlash.c

void OADTarget_getCurrentImageHeader(img_hdr_t *pHdr)

This limitation is resolved by checking for previous incomplete OAD and close the Flash first before proceed to open up the flash again. Highlighted change is shown below in the Profiles\OAD\CC26xx\oad.c file

static uint8_t oadTargetOpened = 0; . . . void OAD_imgIdentifyWrite(uint16_t connHandle, uint8_t *pValue) { img_hdr_t ImgHdr; uint8_t hdrOffset = 0; if (OADTarget_hasExternalFlash()) { hdrOffset = 4; } // Store the new image's header OADTarget_storeImageHeader(pValue); //check if there is a incomplete OAD, then close the Flash first if(oadTargetOpened > 0){ extFlashClose(); } // Read out running image's header. OADTarget_getCurrentImageHeader(&ImgHdr); // Calculate block total of the new image. oadBlkTot = BUILD_UINT16(pValue[hdrOffset + 2], pValue[hdrOffset + 3]) / (OAD_BLOCK_SIZE / HAL_FLASH_WORD_SIZE); oadBlkNum = 0; /* Requirements to begin OAD: * 1) LSB of image version cannot be the same, this would imply a code overlap * between currently running image and new image. * 2) Total blocks of new image must not exceed maximum blocks supported, else * the new image cannot fit. * 3) Block total must be greater than 0. * 4) Optional: Add additional criteria for initiating OAD here. */ if (OADTarget_validateNewImage(pValue + hdrOffset, &ImgHdr, oadBlkTot)) { // Determine where image will be stored. imageAddress = OADTarget_imageAddress(pValue+hdrOffset); imagePage = imageAddress / HAL_FLASH_PAGE_SIZE; // Open the target interface oadTargetOpened = OADTarget_open(); // Image accepted, request block 0. OAD_getNextBlockReq(connHandle, 0); }

About updating device info

Inside PROFILES/devinfoserivice.c

// Firmware Revision String characteristic static uint8 devInfoFirmwareRevProps = GATT_PROP_READ; static uint8 devInfoFirmwareRev[DEVINFO_STR_ATTR_LEN+1] = "V0.02"; // Hardware Revision String characteristic static uint8 devInfoHardwareRevProps = GATT_PROP_READ; static uint8 devInfoHardwareRev[DEVINFO_STR_ATTR_LEN+1] = "RGZ_*"; // Software Revision String characteristic static uint8 devInfoSoftwareRevProps = GATT_PROP_READ; static uint8 devInfoSoftwareRev[DEVINFO_STR_ATTR_LEN+1] = "Ver0.02"; // Manufacturer Name String characteristic static uint8 devInfoMfrNameProps = GATT_PROP_READ; static uint8 devInfoMfrName[DEVINFO_STR_ATTR_LEN+1] = "DongDa";

Change GAP Connection parameters

Write

About porting Sensortag mobile app

Generate Binary file for Android

Create post build script named OffChipOADPostBuild.bat to generate the binary file needed for the OAD, the OFFCHIP_APP_OAD.bin will be what we needed for mobile OAD

This Post Build script will generate

OFFCHIP_APP_OAD.bin for Mobile OAD

OAD_FULL_IMAGE.hex for BLE Device monitor to OAD both Stack and Application

SUPER_FULL_IMAGE.hex to be flashed to chip first time using Flash Programmer 2

@ECHO OFF SET PROJ_DIR=%1 SET BINFILE=OFFCHIP_APP_OAD.bin ECHO project directory is "%PROJ_DIR%" ECHO merge app and stack hex files to one file python "C:\Python27\Scripts\hexmerge.py" -o "%PROJ_DIR%\FlashOnly_OAD_ST_ExtFlash\Exe\OAD_FULL_IMAGE.hex" -r "1000:1CFFF" --overlap=error "%PROJ_DIR%\FlashOnly_OAD_ST_ExtFlash\Exe\SimpleBLEPeripheral_OADExtFlash.hex":1000:EFFF "%PROJ_DIR%\..\..\Stack\CC2640\FlashROM\Exe\SimpleBLEPeripheralStackFlashROM.hex":F000:1CFFF ECHO Generate OAD binary file "%BINFILE%" python "C:\Python27\Scripts\hex2bin.py" -r "1000:BFFF" "%PROJ_DIR%\FlashOnly_OAD_ST_ExtFlash\Exe\SimpleBLEPeripheral_OADExtFlash.hex" "%PROJ_DIR%\FlashOnly_OAD_ST_ExtFlash\Exe\%BINFILE%" ECHO Generate OAD super hex file "SUPER_FULL_IMAGE.hex" python "C:\Python27\Scripts\hexmerge.py" -o "%PROJ_DIR%\FlashOnly_OAD_ST_ExtFlash\Exe\SUPER_FULL_IMAGE.hex" -r "0000:1FFFF" --overlap=error "%PROJ_DIR%\FlashOnly_OAD_ST_ExtFlash\Exe\SimpleBLEPeripheral_OADExtFlash.hex":1000:1EFFF "%PROJ_DIR%\..\..\Stack\CC2640\FlashROM\Exe\SimpleBLEPeripheralStackFlashROM.hex":1000:1EFFF "%PROJ_DIR%\..\..\..\..\..\..\ble\util\BIM_extflash\CC26xx\IAR\FlashOnly\Exe\BIM_ext.hex":0000:1FFFF

Those generated hex files reside in directory

C:\ti\simplelink\ble_cc26xx_2_01_01_44627\Projects\ble\SimpleBLEPeripheral\CC26xx\IAR\Application\CC2640\FlashOnly_OAD_ST_ExtFlash\Exe

Then change the setting in IAR to use this post build script

$PROJ_DIR$\FlashOnly_OAD_ST_ExtFlash\OffChipOADPostBuild.bat $PROJ_DIR$

Security related issue

那么为了防止其他APP 恶意连接，光靠设置配对密码是无效的。

我们一般的做法是，连接成功后需要APP主动发送一包特定的数据给slave，如果slave没有收到这包数据，主动发起断开连接request断开连接。

CC2541之配对与绑定

修改特征值char2在属性表中的属性,

This change make the read action needs bond/pair

GATT_PERMIT_AUTHEN_READ,//GATT_PERMIT_READ,

TI RTOS GPTimer Driver

Pre-release CC26XX / CC13XX PWM and GPTimer driver

Usage - Add the necessary hardware descriptors, objects and configuration to your board files as shown in Board.c/h. - Add a compiler search path to the drivers base or add the drivers to your TI RTOS installation(C:\ti\tirtos_simplelink_2_13_00_06\packages\ti\drivers\timer) - Add the driver source to your workspace - Follow the examples included (main_*.c), there is also an example to trigger timer periodic interrupt inside file GPTimerCC26XX.h

Another Timer PWM example

Local timer library DOC

Customer Board files

Location: C:\ti\tirtos_simplelink_2_13_00_06\packages\ti\boards\CC2640RGZ\CC2650EM_7ID

First make a copy of SRF06EB folder and rename it to our customer board CC2640RGZ,

Change the include path

CCS

IAR

Change the linked Board.c file

CCS

IAR

Flash Simple NV

Open up the stack project, In the predefined symbol, define

OSAL_SNV=2

Inside bcomdef.h

/** @defgroup BLE_NV_IDS BLE Non-volatile IDs * @{ */ // Device NV Items - Range 0 - 0x1F #define BLE_NVID_IRK 0x02 //!< The Device's IRK #define BLE_NVID_CSRK 0x03 //!< The Device's CSRK #define BLE_NVID_SIGNCOUNTER 0x04 //!< The Device's Sign Counter // Bonding NV Items - Range 0x20 - 0x5F - This allows for 10 bondings #define BLE_NVID_GAP_BOND_START 0x20 //!< Start of the GAP Bond Manager's NV IDs #define BLE_NVID_GAP_BOND_END 0x5f //!< End of the GAP Bond Manager's NV IDs Range // GATT Configuration NV Items - Range 0x70 - 0x79 - This must match the number of Bonding entries #define BLE_NVID_GATT_CFG_START 0x70 //!< Start of the GATT Configuration NV IDs #define BLE_NVID_GATT_CFG_END 0x79 //!< End of the GATT Configuration NV IDs // Customer NV Items - Range 0x80 - 0x8F - This must match the number of Bonding entries #define BLE_NVID_CUST_START 0x80 //!< Start of the Customer's NV IDs #define BLE_NVID_CUST_END 0x8F //!< End of the Customer's NV IDs

The osal_snv_read / write can be executed in the same task as the SimpleBLEPeripheral_taskFxn

Or any other tasks registered with ICall.

Sensor Controller

To execute a program on the sensor controller, the program image must first be uploaded to the AUX
RAM by the system CPU or DMA.

The Sensor Controller is set up using a PC-based configuration tool, called Sensor Controller Studio, and
potential use cases may be (but are not limited to):
• Analog sensors using integrated ADC
• Digital sensors using GPIOs, bit-banged I2C, and SPI
• UART communication for sensor reading or debugging
• Capacitive sensing
• Waveform generation
• Pulse counting
• Keyboard scan
• Quadrature decoder for polling rotation sensors
• Oscillator calibration

Pop-ups can also be triggered by moving the cursor to the procedure call and pressing Ctrl+Space.

Data Structure Members

Data structure members are added or edited in a separate pop-up window:

- Data structure: Select one of the following:

- cfg: Used to perform run-time configuration of the task, before the task is started

- input: Used to pass input data to the task (e.g. dynamic parameters for an external sensor)

- output: Used to pass output data to the System CPU application (e.g. accelerometer data)

- state: Internal variables used to store the task's state between iterations

- Member name: Enter the name of the data structure member

- Description: Enter an optional description of the data structure member. This text is included in generated driver source code documentation.

- Type: Select one of the following:

- 16-bit unsigned, decimal

- 16-bit signed, decimal

- 16-bit unsigned, hexadecimal (equivalent to 16-bit unsigned, decimal, but the value is diplayed differently in Sensor Controller Studio)

- 1-bit unsigned, boolean (equivalent to 16-bit unsigned, decimal, but the value is diplayed differently in Sensor Controller Studio)

- Value: Enter a numeric value or an expression using the same syntax as for constant values. For example:

- 123

- PIN_COUNT * 2

- ((A_PARAM + B_PARAM) & 0x003E) | (0x8000 | C_PARAM)

Array size: Select "Not array" for single data structure members, or the name of a user-defined constant for array data structure members

To create a pointer to a data structure member, use the # operator.

Add sensor controller code to existing project

We have set the SCS to generate code inside folder Source/SC_Src

Then we add the path to included directories in both IDEs(IAR, CCS)